Thing that will deduct your mark

| No | Item |
| --- | --- |
| 1 | Board size >10cm * 10cm |
| 2 | Not thick power track (give standard width) |
| 3 | have DRC Error |
| 4 | Crystal not routed correctly (non-symmetrical, too far, wires under) |
| 5 | Wrong bend angle for signal wires |
| 6 | Placing visuals/buttons on the back |
| 7 | Placing XT XH headers on different layers |
| 8 | Each DRC silkscreen warning |
| 9 | Components placed half outside of board |
| 10 | Missing via stitching |
| 11 | No mounting holes |
| 12 | Wires under inductor |
| 13 | Vias on trace are more than 2 |
| 14 | Power across layer without sufficient vias |
| 15 | Protruding components under TFT/back of PCB |
| 16 | TFT floating outside of board |
| 17 | Decoupling capacitor not assigned to each MCU pin |
| 18 | Unnecessary power track loop |
| 19 | Connect stuff in spider-web (not using zone when needed) per group |
| 20 | Labels all over the place (unclear silk screen) |
| 21 | Trace going through the middle part of 2 through-holes (e.g. XH) |
| 22 | FB resistor placed too far |
| 23 | Missing GND plane |
| 24 | Do not show the concept of differential pair signals |
| 25 | Power Track width is not uniform |
| 26 | Missing board name / unclear board name |
| 27 | Tracks too close to the edge |
| 28 | Missing board outline (use wrong layer – not edgecut) |
| 29 |  |
| 30 |  |
| 31 |  |
| 32 |  |

++bonus for board size

| No | Item | Bonus |
| --- | --- | --- |
| 1 | Reduced board size | 10 |
| 2 | Clear silkscreen (SW happie) | 10 |
| 3 | Organised XH header | 10 |

Reminder:

your board is for hand soldering (dont place compoenents too close)